Methodology

For each topology, three hosts are chosen, one server, and two clients. The first client is chosen based on its proximity to the server, representing the best closest case between two nodes on the network. The second client is chosen based on its distance from the server, representing the absolute worst (furthest case). Effectively, this emulates a worst case and best-case scenario which will provide enhanced insight into network topology performances.

Each network is made up of 5 switches, each of which has 3 hosts. This makes a total of 15 hosts able to communicate with each other. To improve realism, each link is given a default set of parameters of 100Mbps, 25ms of delay, 1% loss, and a max queue size of 50. However, our group noticed significant performance issues when running cyclical network topologies. This includes topologies ring, mesh, hybrid, and star. Using the aforementioned link parameters, we observed 90 percent packet loss. This will be further explained in the analysis section; however, for cyclical network tests we ran additional emulations without a restriction on the switch queue size and bandwidth.

To ensure reproducibility, all topologies were constructed using the MiniEdit GUI. Once the topology was complete, the interface was exported to both a python file and a mininet file so that each topology could be tested by each member of our team. These files were stored in a OneDrive folder.

Our test environment uses a VirtualBox instance of Ubuntu 20.04.6 LTS, Mininet 2.3.1b4, and Python 3.8.10 with 8 processing cores and 16GB of ram assigned to it. NOTE: all natively packaged software tools were unmodified except for miniedit.py, where we changed this file to address the inability for the MiniEdit GUI to import files. This is a well-known problem and was resolved by referencing this source.

Once a file is loaded, we start an emulation and open a terminal for our server and two clients. On the server we prepare the iperf3 test with `iperf3 -s` then for each subsequent test we run the following commands (replacing the 10.0.0.1 with the server IP). Two test methods were employed for this.

- Test Linear networks using the MiniEdit tool itself

- Test Cyclical networks by running the exported python files using the mn command and then using xterm on the desired hosts. The MiniEdit tool was used to reference which nodes were representing the best case and worst case (so we would know which hosts to open using xterm to have a server, the closest client to the server, and the furthest client from the server).

The following table illustrates the commands we ran to test our network performance. Notice, two latency test commands were employed.

|  | Bandwidth | Latency | Packet Loss | Jitter |
| --- | --- | --- | --- | --- |
| UDP | iperf3 -c 10.0.0.1 -u -b 100M -t 60 | hping3 10.0.0.1 --udp -p 5201 --fast OR qperf on server, qperf –vvs ‘server_ip’ udp_lat | iperf3 -c 10.0.0.1 -u -b 100M -t 60 | iperf3 -c 10.0.0.1 -u -b 100M -t 60 |
| TCP | iperf3 -c 10.0.0.1 -b 100M -t 60 | hping3 10.0.0.1 -S -p 5201 --fast OR qperf on server, qperf –vvs ‘server_ip’ tcp_lat | N/A | N/A |

The iperf3 tests are run for 60 seconds to give plenty of time for the network to settle and to remove the effect of dramatic outliers. hping3 is allowed to run until 1000 packets are sent for the same reason. When testing UDP bandwidth, connection saturation was considered, thus a full 100Mbps load. However, when testing packet loss and jitter, stressing bandwidth does not accurately reflect network performance for this metric. Thus, the network was not overly saturated, so the test was run with a 1Mbps bandwidth.

Whenever possible, the average value was recorded across the entire test given that an emulation can produce slightly different results. This meant 3-5 tests were run in sequence to ensure that the recorded values were the most accurate.

Linear (h1-h2-h15) - Dev

| Close/Far | Bandwidth | Latency | Packet Loss | Jitter |
| --- | --- | --- | --- | --- |
| UDP | 18.8/16.2Mbps | 118.7/338.3ms | 2.2/6.1% | 1.379/2.032ms |
| TCP | 1,140/287Kbps | 165.2/467.7ms | N/A | N/A |

Linear (h1-h2-h15) - Ben

| Close/Far | Bandwidth | Latency | Packet Loss | Jitter |
| --- | --- | --- | --- | --- |
| UDP | 9.31/7.93 Mbps | 62.8/200ms | 9.1/9.2 % | 0.584/1.795 ms |
| TCP | 1.210/0.206 Mbps | 76.9/500 ms | N/A | N/A |

Ring (h1-h2-h7) - Dev

| Close/Far | Bandwidth | Latency | Packet Loss | Jitter |
| --- | --- | --- | --- | --- |
| UDP | 18.8/18.1Mbps | 118.3/212.8ms | 2.0/3.9% | 1.740/2.236ms |
| TCP | 1,310/355Kbps | 161.3/288.5ms | N/A | N/A |

Ring (h1-h2-h7) - Ben

| Close/Far | Bandwidth | Latency | Packet Loss | Jitter |
| --- | --- | --- | --- | --- |
| UDP | 9.21/8.8 Mbps | 59.1/223 ms | 90.0/91.2% | 2.267 / 2.589 ms |
| TCP | 1.032/0.368 Mbps | 87.4 / 125 ms | N/A | N/A |

Ring (h1-h2-h7) - Ben Cyclical Resolution

| Close/Far | Bandwidth | Latency | Packet Loss | Jitter |
| --- | --- | --- | --- | --- |
| UDP | 97.8/95.6 Mbps | 52.9 / 111 ms | 2 / 4 % | 0.094 / 0.059 ms |
| TCP | 1.34 / 0.472 Mbps | 52.6 / 167 ms | N/A | N/A |

Mesh (h1-h2-h7) - Dev

| Close/Far | Bandwidth | Latency | Packet Loss | Jitter |
| --- | --- | --- | --- | --- |
| UDP | 18.4/18.0Mbps | 115.1/173.8ms | 2.2/3.1% | 1.557/2.748ms |
| TCP | 1,290/689Kbps | 159.7/242.5ms | N/A | N/A |

Mesh (h1-h2-h7) - Ben

| Close/Far | Bandwidth | Latency | Packet Loss | Jitter |
| --- | --- | --- | --- | --- |
| UDP | 8.6 / 8.3Mbps | 62.8 / 126 ms | 89.2 / 91.2 % | 0.988 / 1.002 ms |
| TCP | 1.09 /0.368 Mbps | 74.1 / 200 ms | N/A | N/A |

Mesh (h1-h2-h7) - Ben Cyclic Resolution

| Close/Far | Bandwidth | Latency | Packet Loss | Jitter |
| --- | --- | --- | --- | --- |
| UDP | 97.5 / 96.7 Mbps | 52.6 / 83.3 ms | 2 / 3 % | 0.081 / 0.062 ms |
| TCP | 1.30 / .975 Mbps | 62.5 / 100 ms | N/A | N/A |

Star (h4-h5-h10) - Dev

| Close/Far | Bandwidth | Latency | Packet Loss | Jitter |
| --- | --- | --- | --- | --- |
| UDP | 18.7/17.4Mbps | 118.5/205.2ms | 2.4/3.7% | 1.480/2.002ms |
| TCP | 1,320/378Kbps | 157.7/309.1ms | N/A | N/A |

Star (h4-h5-h10) - Ben

| Close/Far | Bandwidth | Latency | Packet Loss | Jitter |
| --- | --- | --- | --- | --- |
| UDP | 9.70 / 9.01Mbps | 60.6 / 155 ms | 91 / 92% | 0.742 ms / 2.161 |
| TCP | 1.19 / .502 Mbps | 69 / 200 ms | N/A | N/A |

Star (h4-h5-h10) - Ben Cyclic Resolution

| Close/Far | Bandwidth | Latency | Packet Loss | Jitter |
| --- | --- | --- | --- | --- |
| UDP | 97.7 / 95.6 Mbps | 52.6 / 201 ms | 2 / 4% | 0.050 / 0.012ms |
| TCP | 1.28 / 0.472 Mbps | 62.5 / 333 ms | N/A | N/A |

Tree (h7-h8-h10) - Dev

| Close/Far | Bandwidth | Latency | Packet Loss | Jitter |
| --- | --- | --- | --- | --- |
| UDP | 18.5/17.1Mbps | 116.5/262.8ms | 2.2/4.7% | 1.669/2.388ms |
| TCP | 1,260/312Kbps | 163.7/408.9ms | N/A | N/A |

Tree (h1-h2-h15) - Ben

| Close/Far | Bandwidth | Latency | Packet Loss | Jitter |
| --- | --- | --- | --- | --- |
| UDP | 9.67/9.65 Mbps | 80.4 /167 ms | 9.0/9.0 % | 2.401/2.753 ms |
| TCP | 1.11 /0.293 Mbps | 83.5 / 335 ms | N/A | N/A |

Hybrid (h1-h2-h15) - Dev

| Close/Far | Bandwidth | Latency | Packet Loss | Jitter |
| --- | --- | --- | --- | --- |
| UDP | 18.5/17.3Mbps | 117.2/211.6ms | 2.1/4.1% | 1.890/1.902ms |
| TCP | 1,240/440Kbps | 163.6/283.0ms | N/A | N/A |

Hybrid (h1-h2-h15) - Ben

| Close/Far | Bandwidth | Latency | Packet Loss | Jitter |
| --- | --- | --- | --- | --- |
| UDP | 9.05 / 8.83 Mbps | 62.8 / 167 ms | 91 / 91 % | 1.053 / 2.687ms |
| TCP | 1.29 / 0.353 Mbps | 63.1 / 200 ms | N/A | N/A |

Hybrid (h1-h2-h15) - Ben Cyclic Resolution

| Close/Far | Bandwidth | Latency | Packet Loss | Jitter |
| --- | --- | --- | --- | --- |
| UDP | 97.8 / 94.5 Mbps | 52.6 / 143 ms | 2 / 5% | 0.125 / 0.084 ms |
| TCP | 1.33 / 0.321 Mbps | 62.5 / 250 ms | N/A | N/A |